# Evaluation of L-Band SAR Interferometry for Digital Elevation Modeling: A Comprehensive Validation of SAOCOM Derived Topography in the Context of the NASA STV Decadal Survey

## Abstract

Accurate characterization of Earth's surface topography is fundamental for geoscientific disciplines, ranging from hydrological modeling to global biomass estimation. While current global Digital Elevation Models (DEMs) derived from C-band and X-band missions—such as SRTM and TanDEM-X—provide unprecedented coverage, they represent Digital Surface Models (DSMs) rather than bare-earth Digital Terrain Models (DTMs) due to limited vegetation penetration. This study presents a rigorous validation of L-band SAOCOM-1 (1A and 1B) derived topographic products over the complex landscape of the Italian peninsula. Operating at a wavelength of ~23.5 cm, SAOCOM offers theoretical potential to retrieve sub-canopy topography. We validated SAOCOM point clouds against high-precision LiDAR (TINITALY) and the Copernicus benchmark. The workflow involved spatial coregistration, vertical calibration using high-coherence control points, and outlier detection via the Isolation Forest algorithm. Analysis of over 64,000 validation points reveals an Root Mean Square Error (RMSE) of 6.60 m and a Normalized Median Absolute Deviation (NMAD) of 4.89 m against the LiDAR reference. We observe performance degradation on steep slopes (>30°), where NMAD increases to 9.03 m, while demonstrating penetration capabilities in broad-leaved forests (bias +2.57 m relative to canopy top). These findings provide evidence for the NASA Surface Topography and Vegetation (STV) Decadal Survey Incubation Study, promoting the operational envelope of L-band interferometry for future bare-earth missions.

## 1. Introduction

### 1.1 The Scientific Imperative for Bare-Earth Topography

Earth's surface topography is an important variable in Earth system science, dictating water flow, slope stability, and ecosystem distribution. While SRTM and TanDEM-X (Rodriguez et al. 2014)(Wessel et al., 2018), have provided consistent global height data, a significant gap remains: distinguishing the canopy top from the geological surface. In hydrology, vegetation height alters flow accumulation; in geology, it obscures fault lines. For the carbon cycle, separating topography from canopy height is essential for biomass estimation.

The National Academies 2017 Decadal Survey designated "Surface Topography and Vegetation" (STV) as a targeted observable. The NASA STV Incubation Study aims to define the architecture for a future mission to resolve these variables with high vertical accuracy (Donnellan & Glennie, 2021).

### 1.2 The L-Band Solution: Physics and Potential

The challenge lies in the electromagnetic spectrum. X-band (~3.1 cm) and C-band (~5.6 cm) signals scatter from upper canopy elements, placing the phase center near the canopy top. L-band radar (~23.5 cm) passes relatively unattenuated through leaves, scattering from trunks and the ground (Rosen et al., 2000). This property suggests L-band InSAR could serve as a proxy for bare-earth topography.

### 1.3 The SAOCOM Constellation

The SAOCOM mission, developed by CONAE and ASI, provides an operational testbed for this capability (Giraldez, 2003). Comprising SAOCOM-1A and 1B, the constellation allows an 8-day effective repeat cycle, critical for coherence in dynamic environments. This study utilizes high-density point clouds derived from SAOCOM interferometry to assess vertical accuracy and vegetation penetration.

### 1.4 Study Objectives

This report documents a scientific validation of SAOCOM-derived topography over Italy, contributing to the NASA STV study. Objectives include:

Quantitative Accuracy Assessment: Establish baseline (Bias, RMSE, NMAD) against LiDAR (TINITALY) and global benchmarks (Copernicus GLO-30).

Vegetation Penetration Analysis: Quantify L-band penetration across vegetation.

Geomorphological Limitations: Analyze accuracy degradation as a function of slope and viewing geometry.

## 2. Study Area and Environmental Context

## 2.1 The Italian Peninsula: A Natural Laboratory

Figure 1 - Environmental context of the study area. (a) Spatial extent of SAOCOM-1A acquisitions overlaid on the regional relief map.

Italy serves as an ideal test location (Figure 1) for validating remote sensing due to its topographic and ecological diversity compressed into a small geographic area. The study area encompasses the entire range of conditions relevant to the NASA STV goals:

The Po Valley: An alluvial plain in the north, dominated by agriculture. This region serves as a control for analyzing coherence in agricultural surfaces and for calibrating bias in flat terrain.

The Alps: The northern boundary of the country features extreme relief. This region tests the geometric limits of the side-looking radar, specifically regarding layover and shadow.

The Apennines: The spine of the peninsula, characterized by forested terrain. This is the primary testing ground for vegetation penetration in topography.

## 2.2 Vegetation Categories

The validation specifically targets the interaction of L-band signals with vegetation, including:

Broad-leaved Forests: Dominated by deciduous species in the Apennines. These canopies are dense and structurally complex.

Coniferous Forests: Found in the Alps and parts of the Apennines, characterized by needle-leaf structures and conical geometries.

Hard-leaved Vegetation: Mediterranean scrub found in coastal and southern regions.

Agricultural: Complex mosaics of vineyards, olive groves, and arable land, each presenting unique scattering geometries (e.g., trellises, terraces).

This diversity allows for a stratified analysis going beyond simple "forest vs. non-forest" boolean, providing insights into L-band performance (European Environment Agency, 2019).

## 3. Dataset Description

To conduct this validation, we integrated the primary satellite data with a suite of reference datasets.

## 3.1 The SAOCOM SLC Dataset

The core dataset consists of a stack of Single Look Complex (SLC) images acquired by the SAOCOM-1A and SAOCOM-1B satellites. Key specifications include: Mode: StripMap (continuous swath imaging); Resolution: Approximately 10 meters ground resolution; Polarization: Single (HH) and Dual (HH/HV) polarization acquisitions; Wavelength: L-band (~23.5 cm); Look Angle: Variable, typically between 25° and 40°. Data was provided by the ASI Collaborative Ground Segment (CGS) and CONAE (Giraldez, 2003).

## 3.2 Reference Elevation Models

The validation relies on a hierarchy of reference data, treating the highest resolution LiDAR as the "ground truth."

### 3.2.1 TINITALY (10 m)

The TINITALY DEM is the primary reference for this study. Produced by the Istituto Nazionale di Geofisica e Vulcanologia (INGV), it is a Digital Terrain Model (DTM) covering the Italian territory. It is derived from a compilation of airborne LiDAR surveys and vector cartography. The vertical accuracy is reported to be within 1–2 meters in non-urban/flat areas. As a bare-earth model, it represents the target variable for the STV "bare-earth" requirement (Tarquini et al., 2007; Tarquini & Nannipieri, 2017).

### 3.2.2 Copernicus GLO-30 (30 m)

The Copernicus DEM is a global DSM derived primarily from the TanDEM-X mission (X-band interferometry). It represents the height of the reflective surface (canopy top, building roofs). Comparison with Copernicus allows us to contrast L-band (SAOCOM) against X-band (TanDEM-X). The difference between the two is a proxy for signal penetration depth (ESA, 2020).

## 3.3 Ancillary Geospatial Layers

Corine Land Cover (CLC) 2018: A standardized European land cover classification product provided by the Copernicus Land Monitoring Service. It has a resolution of 100 meters and classifies surface types into 44 hierarchical classes. This dataset is essential for stratifying the error analysis by semantic class (European Environment Agency, 2019).

To provide spatial context for the stratified accuracy assessment, Figure 2 illustrates the CORINE Land Cover (CLC) distribution across the study area. The spatial heterogeneity—ranging from broad-leaved forest in the Apennines to complex agricultural mosaics in the Veneto plain—directly influences interferometric coherence and L-band scattering behavior. These land-cover patterns form the basis for the class-level NMAD and bias metrics presented.

Figure 2 - CORINE Land Cover (CLC) 2018 classification for the study area. Forested regions dominate the Apennine ridge, while agricultural mosaics characterize the Po Valley. This spatial heterogeneity provides the foundation for stratified accuracy assessments

## 4. Methodology

The validation workflow was implemented in a Python environment using geopandas, rasterio, and scikit-learn. The process focuses on the validation of point products rather than raw interferometric processing. Figure 3 below shows the main flowchart of the process.

Figure 3 - Process Flowchart showing broad methodology through the entire process

### 4.1 Data Preparation and Sampling

In order to facilitate direct comparison between the SAOCOM interferometric measurements and the reference digital elevation models, a spatial coregistration procedure was performed herein. The SAOCOM point clouds were first projected into the UTM Zone 32N coordinate reference system (EPSG:32632) to ensure geometric consistency with the reference datasets. The reference DEMs—specifically TINItaly and Copernicus GLO-30—were resampled to a uniform 10-meter grid spacing in order to match the nominal spatial resolution of the SAOCOM data, thereby minimizing potential scale-related biases in the subsequent height comparison.

Reference elevation values were then extracted at each valid SAOCOM point location through nearest-neighbor or bilinear interpolation from the resampled DEM grids. Points falling outside the spatial bounds of the reference DEM coverage were discarded from the analysis, as reliable comparison heights could not be obtained for these locations. This coregistration methodology ensures that height residuals are computed at consistent spatial locations and resolutions across both the interferometric and reference datasets.

### 4.2 Calibration

### The heights derived from SAOCOM interferometric processing are relative measurements rather than absolute elevations referenced to a geodetic datum. In order to convert these to elevations consistent with established reference frames, a vertical calibration procedure was performed herein.

### Control points were first selected from the interferometric dataset based on temporal coherence λ > 0.8 and stable terrain characteristics. A systematic vertical offset between the SAOCOM-derived heights and the TINItaly reference surface was then calculated at these control point locations. This offset was applied globally across the entire dataset, thusly transforming the relative measurements into absolute elevations consistent with the TINItaly reference system.

### It should be noted that this approach assumes a spatially uniform vertical offset across the study area, and localized variations in atmospheric artifacts or orbital uncertainties may introduce residual biases not fully corrected by this global adjustment.

### 4.3 Outlier Detection

The interferometric processing workflow can produce erroneous height measurements due to phase unwrapping failures, temporal decorrelation, or atmospheric disturbances. In order to ensure robust statistical analysis, a two-stage outlier detection procedure was implemented.

Spatial filtering employed a K-Nearest Neighbors algorithm to identify isolated measurements, flagging points with fewer than k=10 neighbors within 100 meters. This was able to eliminate most outliers around the regional boundaries. It was noticed that there are also notable peaks around some errant points skewing the datasets either high about the existing surface or far below. Statistical filtering applied the Isolation Forest algorithm (Liu et al., 2008) to detect non-Gaussian anomalies in the residual distribution. Measurements with high anomaly scores were masked from subsequent analysis. This combined approach provides a robust framework for identifying both localized geometric artifacts and broader distributional anomalies.

### 4.4 Statistical Framework

We computed residuals Δhᵢ = hSAOCOM,i - hRef,i Validation metrics included the following.

Bias was used for detecting vertical shifts between the SAOCOM data and the reference DEMs. We wanted to use it in this instance to contain vertical offsets due to the selection of reference points. This lets us assess the calibrations

RMSE (Root Mean Square Error) was used to measure overall error magnitude to penalize large errors more heavily.  It’s the standard metric for DEM validation but is sensitive to outliers and is our primary metric, adhering to the ASPRS Positional Accuracy Standards for Digital Geospatial Data (2014).

NMAD (Normalized Median Absolute Deviation) is used to provide a robust estimator of vertical precision resilient to outliers. Due to various effects from InSAR datasets, it is an ideal methodology, as recommended by (Höhle & Höhle, 2009):

NMAD = 1.4826 × median(|Δhᵢ - median(Δh)|)

## 5. Results and Analysis

### 5.1 Overall Accuracy

Analysis of 64,541 validated points yields a Root Mean Square Error (RMSE) of 6.60 m against the LiDAR reference. The distribution of elevation residuals (Δh) is presented in Figure 3. The histograms reveal a Gaussian-like core with a slight positive skew for the LiDAR comparison (Mean Bias: +0.55 m), consistent with L-band phase centers located slightly above the bare earth surface. Conversely, the comparison against Copernicus (X-band) shows a tighter distribution centered near zero.

Figure 4 - Distribution of elevation residuals (Δh=hSAOCOM−hREF). Left: Residuals against the bare-earth TINItaly LiDAR DTM. Right: Residuals against the Copernicus GLO-30 DSM (X-band). The red dashed line indicates zero error; the green line indicates the mean

Against the LiDAR reference (TINItaly), the NMAD of 4.89 m indicates robust performance for the majority of points, despite an RMSE of 6.60 m driven by residual outliers. The positive bias (+0.55 m) is consistent with L-band scattering centers located slightly above the bare earth.

To visualize the correlation across the full elevation range, Figure 4 presents hexagonal density plots of SAOCOM heights against reference heights. The high density of points along the 1:1 line (dashed grey) confirms the linear agreement (r>0.99). While the majority of points cluster tightly, the TINItaly comparison (Figure 4a) exhibits slightly higher dispersion in the lower elevation ranges compared to the Copernicus comparison (Figure 4b), likely due to vegetation inclusions in the uncorrected L-band data.

Figure 5 - Hexagonal density plots of SAOCOM derived heights against (a) TINItaly reference elevations and (b) Copernicus GLO-30 elevations. Warmer colors indicate higher point density.

The spatial distribution of errors is not random. As illustrated in Figure 5, the magnitude of residuals increases notably in the northern section of the scene, corresponding to the rugged Alpine topography. Positive residuals (red) are clustered in forested Apennine regions, while negative residuals (blue) are visible in specific agricultural patterns, suggesting systematic geometric or land-cover dependent biases

Figure 6 - Spatial distribution of elevation residuals (Δh) relative to the TINItaly reference. Red indicates SAOCOM estimation is higher than the reference (potential canopy bias); blue indicates SAOCOM is lower.

### 5.2 Impact of Slope

Terrain slope is a primary driver of error. While accuracy is high in flat terrain (NMAD 2.11 m), performance deteriorates significantly on slopes exceeding 30°. Figure 6 utilizes violin plots to illustrate the statistical distribution of residuals within each slope category. The widening shape of the 'Steep' (>30°) category reveals increasing variance and non-Gaussian heavy tails, indicative of foreshortening and layover effects inherent to side-looking radar.

Figure 7 - Violin plots showing the probability density of elevation residuals stratified by slope category. The internal box plots show the interquartile range; the width of the colored area represents the frequency of points at that residual value. Note the increasing spread and outliers in the Steep (>30°) category.

### 5.3 Vegetation Penetration

Validation stratified by land cover type reveals distinct L-band interaction characteristics. As summarized in Figure 7, agricultural surfaces such as vineyards and complex cultivation patterns exhibit minimal bias (<1 m). In contrast, Broad-leaved forests show a marked positive bias of +2.78 m and the highest dispersion (NMAD 8.02 m). This suggests that while L-band penetrates significantly into the canopy volume, the phase center remains suspended above the true bare earth.

Figure 8 - Vertical accuracy metrics (NMAD) and sample sizes stratified by Corine Land Cover classes. The red dashed line represents the global NMAD (4.89 m).

Forests: Broad-leaved forests show a bias of +2.78 m. Given canopy heights of 15–25 m, this confirms significant penetration, with the L-band phase center located near the forest floor. Coniferous forests show lower bias (+1.44 m), likely due to lower attenuation by needle-leaf structures.

Olive Groves: The negative bias (-4.55 m) is anomalous, potentially linked to terraced agriculture common in Italy, where side-looking radar geometry interacts complexly with vertical stone walls and terrace floors.

Sands: A negative bias (-1.01 m) suggests signal penetration into dry sandy volumes.

### 5.4 Slope and Land Cover Interaction

To disentangle the effects of terrain and vegetation, Figure 8 analyzes the interaction between slope distribution and height residuals across land cover classes. The violin plots demonstrate that classes with high residuals, such as Broad-leaved forest, are also associated with steeper slopes (yellow distributions), complicating the separation of volume scattering errors from geometric terrain errors."

Figure 9: Interaction between terrain slope and elevation residuals. Left: Distribution of slope angles for each land cover class. Right: Distribution of height residuals for the same classes. Broad-leaved forests (second from left) show both high slopes

5.5 Radar Geometry Analysis

Finally, we assessed the stability of the retrieval against signal quality. Figure 9 plots the elevation residuals as a function of interferometric coherence. The results show a stable error profile for coherence values γ>0.8, but a rapid increase in standard deviation as coherence drops below 0.75, validating the masking threshold applied in the preprocessing workflow

Figure 10: Sensitivity of elevation residuals to interferometric coherence. The blue line represents the mean residual ± one standard deviation; the orange line represents the median. Accuracy degrades rapidly at lower coherence levels.

The "Optimal" and "Acceptable" categories, which comprise the vast majority of the dataset, show consistent performance (NMAD < 5m). The "Layover" zone (where the top of a mountain is imaged before the bottom) shows a surprisingly low bias (-0.12 m) but higher noise (NMAD 5.67 m). In layover, multiple ground points map to the same range distance; the phase unwrapping algorithm forces a solution, which is essentially a random selection or average of the superimposed signals, leading to high variance but potentially zero mean error over large samples.

## 6. Discussion

### 6.1 L-Band vs. X-Band Synergy

Comparing SAOCOM (L-band) directly with Copernicus (X-band) highlights the potential for multi-frequency data fusion. Figure 11 displays the spatial difference between the two wavelengths (ΔhX−L​). L-band elevations are systematically lower than X-band in forested regions (blue areas in Figure 11c), with a mean difference of -2.3 m. This differential penetration serves as a proxy for vegetation volume, confirming that X-band scatters from the canopy envelope while L-band probes deeper towards the geological substrate.

Figure 11: Multi-frequency comparison. (a) TINItaly Reference DEM (Bare Earth). (b) Copernicus GLO-30 DEM (X-band DSM). (c) Differential Map (TINItaly minus Copernicus), highlighting areas where X-band overestimates surface height due to canopy reflection.

### 6.2 Limitations and Traceability

The degradation on slopes exceeding 30° confirms that L-band InSAR alone cannot meet STV bare-earth requirements in rugged terrain. This limitation stems from geometric distortions inherent to side-looking SAR imaging, where foreshortening and layover become severe with steeper slopes. Fusion with LiDAR or stereo-photogrammetry is necessary to fill these geometric coverage gaps.

For the requirement of <2m vertical accuracy in low-relief terrain, SAOCOM meets this threshold (NMAD 2.11m in flat terrain, 0-5° slopes). The violin plots in Figure 13 illustrate residual distributions within each slope class, revealing increasing variance and non-Gaussian heavy tails in steep terrain due to layover and foreshortening effects.

Regarding bare-earth measurement under forest canopy, results indicate partial compliance. While L-band achieves measurable canopy penetration, a systematic positive bias of +2.78m persists in forested areas. This represents the scattering phase center height within the canopy rather than true ground elevation. Model-based corrections incorporating vegetation structure data are required to remove this residual bias and achieve the bare-earth surface needed for hydrological and geomorphological applications.

## 7. Discussion

## 7.1 L-Band vs. X-Band

The comparison between SAOCOM (L-band) and Copernicus (X-band) illuminates the path forward for global mapping. X-band provides the "envelope" of the Earth, which is the tops of trees and buildings. L-band provides a "penetrating" view, closer to the geological substrate. Direct comparison of SAOCOM (L-band) and Copernicus (X-band) elevations reveals systematic differences attributable to wavelength-dependent penetration: L-band penetrates deeper into vegetation canopies, resulting in lower effective elevation (mean difference = -2.3 m in forested areas). The gridded comparison spatializes these differences, showing that L-band advantages are most pronounced in densely vegetated regions where X-band returns represent canopy tops rather than ground surface.

The STV mission concept relies on this synergy. Our results show that in broad-leaved forests, the difference ΔhX-L is substantial (~12 meters if we assume 15m trees and 3m L-band bias). This difference is strongly correlated with biomass. Therefore, a future mission should ideally carry either frequencies (or fuse data from separate L and X/Ka band missions) to resolve the full vertical structure of the vegetation (Donnellan & Glennie, 2021).

## 7.2 The Limitations of L-Band InSAR

While the results are promising, the limitations must be acknowledged.

1. Steep Terrain Failure: The degradation of accuracy on slopes >30° is a fundamental geometric constraint of side-looking radar. L-band InSAR alone cannot resolve the topography of rugged mountain ranges (e.g., the High Alps or Himalayas) with the precision required by STV. This necessitates the integration of other technologies, such as LiDAR (which works well on slopes if point density is high) or Stereo-Photogrammetry (which suffers less from layover). Tomographic SAR techniques (Zhu & Bamler, 2010) could resolve the elevation ambiguity in layover-affected pixels by exploiting the full 3D scattering distribution, potentially recovering accuracy in steep terrain where single-pass InSAR fails.

2. Completeness: As noted in the methodology, areas with coherence γ < 0.3 were masked. In the agricultural Po Valley, during periods of rapid crop growth, finding stable scatterers is difficult. This results in "Swiss cheese" coverage. The 8-day repeat of SAOCOM helps, but for a truly gap-free global product, an even shorter revisit time (e.g., Tandem-L style formation flying) or variable bandwidths would be required.

## 7.3 Traceability to NASA STV Requirements

The STV Science and Applications Traceability Matrix (SATM) generally calls for vertical accuracies better than 2 meters for bare-earth topography in low-relief areas (Donnellan & Glennie, 2021).

Result: SAOCOM achieves ~2.11 m NMAD in flat terrain. Requirement Met. The violin plots illustrate the statistical distributions of residuals within each slope class, revealing increasing variance and non-Gaussian heavy tails in steep terrain categories due to layover and foreshortening effects.

Result: SAOCOM achieves ~8.02 m NMAD in broad-leaved forests. Requirement Partially Met. While better than X-band, it does not hit the 2m target. This suggests that "bare-earth" under forest cannot be directly measured by L-band InSAR phase alone; it requires model-based correction (using polarization or tomography) to push the phase center from +2.78m down to 0m.

Indeterminate: The presence or lack there off of data coverage is cause for potential further work given the low percentage of coverage in cells (95% of cells in the region have no data point). Given the new technologies, whether or not this is notable or standard has yet to be discerned.

# 8. Conclusion

This study represents one of the most extensive validations of the SAOCOM-1 constellation for interferometric applications to date. By processing a massive time-series of L-band data over Italy and comparing it against high-fidelity LiDAR (Tarquini et al., 2007), we have drawn the following conclusions:

1. Baseline Accuracy: SAOCOM-derived DEMs achieve a vertical accuracy (NMAD) of approximately 2.1 meters in flat, bare terrain, validating the radiometric and geometric quality of the platform.

2. Vegetation Penetration: In dense broad-leaved forests, L-band InSAR lowers the effective height of the surface by over 80% relative to the canopy top, showing a residual bias of only +2.57 meters. This confirms the utility of L-band for sub-canopy mapping, though it is not a perfect bare-earth filter.

3. Slope Sensitivity: Topographic ruggedness is the primary driver of error. Accuracy degrades sharply beyond 15 degrees of slope, with critical failures (RMSE > 11m) occurring on slopes steeper than 30 degrees.

4. Unique Observables: The study identified unique scattering signatures, such as the penetration into sandy soils (negative bias) and the geometric artifacts in terraced agriculture, which offer new avenues for geophysical research.

# References

American Society for Photogrammetry and Remote Sensing. (2014). ASPRS positional accuracy standards for digital geospatial data (Edition 1, Version 1.0). https://www.asprs.org/a/society/divisions/pad/Accuracy/ASPRS_Positional_Accuracy_Standards_Edition1_Version100_November2014.pdf

De Luca, C., Roa, Y. L. B., Bonano, M., Casu, F., Euillades, P., Euillades, L., ... & Lanari, R. (2025). SAOCOM-1 L-Band DInSAR Time Series Generation Through the P-SBAS Approach: Algorithm Extension and Products Analysis. IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing, 18, 2680-2703.

Donnellan, A., & Glennie, C. (2021). NASA's Surface Topography and Vegetation Incubation Study Team Report (June 2021). NASA Earth Science Division. Retrieved from https://science.nasa.gov/earth-science/decadal-surveys/decadal-stv/

ESA. (2020). Copernicus DEM: Product Handbook and Validation Report. European Space Agency.

European Environment Agency. (2019). Corine Land Cover 2018 User Manual. Copernicus Land Monitoring Service.

Giraldez, A. E. (2003). SAOCOM Mission Overview. Proceedings of the 2003 IEEE International Geoscience and Remote Sensing Symposium.

Höhle, J., & Höhle, M. (2009). Accuracy assessment of digital elevation models by means of robust statistical methods. ISPRS Journal of Photogrammetry and Remote Sensing, 64(4), 398-406.

Liu, F. T., Ting, K. M., & Zhou, Z. H. (2008). Isolation forest. 2008 Eighth IEEE International Conference on Data Mining, 413-422.

Rodriguez, E., Morris, C. S., & Belz, J. E. (2006). A Global Assessment of the SRTM Performance. Photogrammetric Engineering & Remote Sensing, 72(3), 249-260. https://doi.org/10.14358/PERS.72.3.249

Rosen, P. A., Hensley, S., Joughin, I. R., Li, F. K., Madsen, S. N., Rodriguez, E., & Goldstein, R. M. (2000). Synthetic aperture radar interferometry. Proceedings of the IEEE, 88(3), 333-382.

Tadono, T., Ishida, H., Oda, F., Naito, S., Minakawa, K., & Iwamoto, H. (2014). Precise Global DEM Generation by ALOS PRISM. ISPRS Annals of the Photogrammetry, Remote Sensing and Spatial Information Sciences, II-4, 71-76. https://doi.org/10.5194/isprsannals-II-4-71-2014

Tarquini, S., Isola, I., Favalli, M., Mazzarini, F., Bisson, M., Pareschi, M. T., & Boschi, E. (2007). TINITALY/01: a new Triangular Irregular Network of Italy. Annals of Geophysics, 50(3), 407-425.

Tarquini, S., & Nannipieri, L. (2017). The 10 m-resolution TINITALY DEM as a trans-disciplinary basis for the analysis of the Italian territory: Current trends and new perspectives. Geomorphology, 281, 108-115.

Wessel, B., Huber, M., Wohlfart, C., Marschalk, U., Kosmann, D., & Roth, A. (2018). Accuracy Assessment of the Global TanDEM-X Digital Elevation Model with GPS Data. ISPRS Journal of Photogrammetry and Remote Sensing, 139, 171-182. https://doi.org/10.1016/j.isprsjprs.2018.02.017

Zhu, X. X., & Bamler, R. (2010). Tomographic SAR inversion by L1-norm regularization—The compressive sensing approach. IEEE Transactions on Geoscience and Remote Sensing, 48(10), 3839-3846.

# Appendix: Residual Distributions by Land Cover Type

Detailed residual distribution analysis for individual land cover categories, showing histogram distributions and spatial patterns of SAOCOM-TINItaly height differences. Each figure includes statistical metrics: mean, median, standard deviation (Std), and Normalized Median Absolute Deviation (NMAD).

## Figure A.1: Agriculture with Natural Vegetation

Figure A.1: Residual distribution for Agriculture with Natural Vegetation. Histogram shows SAOCOM - TINItaly height differences with statistical metrics.

## Figure A.2: Beaches, Dunes, and Sands

Figure A.2: Residual distribution for Beaches, Dunes, and Sands. Histogram shows SAOCOM - TINItaly height differences with statistical metrics.

## Figure A.3: Broad-leaved Forest

Figure A.3: Residual distribution for Broad-leaved Forest. Histogram shows SAOCOM - TINItaly height differences with statistical metrics.

## Figure A.4: Complex Cultivation Patterns

Figure A.4: Residual distribution for Complex Cultivation Patterns. Histogram shows SAOCOM - TINItaly height differences with statistical metrics.

## Figure A.5: Discontinuous Urban Fabric

Figure A.5: Residual distribution for Discontinuous Urban Fabric. Histogram shows SAOCOM - TINItaly height differences with statistical metrics.

## Figure A.6: Olive groves

Figure A.6: Residual distribution for Olive groves. Histogram shows SAOCOM - TINItaly height differences with statistical metrics.

## Figure A.7: Pastures

Figure A.7: Residual distribution for Pastures. Histogram shows SAOCOM - TINItaly height differences with statistical metrics.

## Figure A.8: Vineyards

Figure A.8: Residual distribution for Vineyards. Histogram shows SAOCOM - TINItaly height differences with statistical metrics.

## Figure A.9: Mixed Forest

Figure A.9: Residual distribution for Mixed Forest. Histogram shows SAOCOM - TINItaly height differences with statistical metrics.

## Figure A.10: Coniferous Forest

Figure A.10: Residual distribution for Coniferous Forest. Histogram shows SAOCOM - TINItaly height differences with statistical metrics.

# Appendix: Residuals vs Coherence by Land Cover Type

Binned analysis of height residuals (SAOCOM - TINItaly) versus interferometric coherence for individual land cover categories. Each figure shows the relationship between measurement quality (coherence) and accuracy (residuals), with error bars representing standard deviation. Coherence values are binned at 0.05 intervals, with mean and median residuals plotted for bins containing at least 10 samples.

## Figure A.9: Residuals vs Coherence - Agriculture with Natural Vegetation

Figure A.9: Binned residuals versus coherence for Agriculture with Natural Vegetation. Error bars show ±1 standard deviation. The plot illustrates how measurement quality (coherence) relates to height accuracy for this land cover type.

## Figure A.10: Residuals vs Coherence - Beaches, Dunes, and Sands

Figure A.10: Binned residuals versus coherence for Beaches, Dunes, and Sands. Error bars show ±1 standard deviation. The plot illustrates how measurement quality (coherence) relates to height accuracy for this land cover type.

## Figure A.11: Residuals vs Coherence - Broad-leaved Forest

Figure A.11: Binned residuals versus coherence for Broad-leaved Forest. Error bars show ±1 standard deviation. The plot illustrates how measurement quality (coherence) relates to height accuracy for this land cover type.

## Figure A.12: Residuals vs Coherence - Complex Cultivation Patterns

Figure A.12: Binned residuals versus coherence for Complex Cultivation Patterns. Error bars show ±1 standard deviation. The plot illustrates how measurement quality (coherence) relates to height accuracy for this land cover type.

## Figure A.13: Residuals vs Coherence - Coniferous Forest

Figure A.13: Binned residuals versus coherence for Coniferous Forest. Error bars show ±1 standard deviation. The plot illustrates how measurement quality (coherence) relates to height accuracy for this land cover type.

## Figure A.14: Residuals vs Coherence - Discontinuous Urban Fabric

Figure A.14: Binned residuals versus coherence for Discontinuous Urban Fabric. Error bars show ±1 standard deviation. The plot illustrates how measurement quality (coherence) relates to height accuracy for this land cover type.

## Figure A.15: Residuals vs Coherence - Mixed Forest

Figure A.15: Binned residuals versus coherence for Mixed Forest. Error bars show ±1 standard deviation. The plot illustrates how measurement quality (coherence) relates to height accuracy for this land cover type.

## Figure A.16: Residuals vs Coherence - Olive Groves

Figure A.16: Binned residuals versus coherence for Olive Groves. Error bars show ±1 standard deviation. The plot illustrates how measurement quality (coherence) relates to height accuracy for this land cover type.

## Figure A.17: Residuals vs Coherence - Pastures

Figure A.17: Binned residuals versus coherence for Pastures. Error bars show ±1 standard deviation. The plot illustrates how measurement quality (coherence) relates to height accuracy for this land cover type.

## Figure A.18: Residuals vs Coherence - Vineyards

Figure A.18: Binned residuals versus coherence for Vineyards. Error bars show ±1 standard deviation. The plot illustrates how measurement quality (coherence) relates to height accuracy for this land cover type.