# SAOCOM DEM Validation Analysis - Comprehensive Report

Generated: 2025-12-04 11:43:12

# 1. Overall Validation Statistics

Comparison of SAOCOM heights against reference DEMs. Metrics computed after calibration and outlier removal.

| Reference DEM | N Points | Bias (m) | RMSE (m) | NMAD (m) |
| --- | --- | --- | --- | --- |
| TINItaly | 64,541 | +0.55 | 6.60 | 4.89 |
| Copernicus | 64,649 | +0.04 | 7.17 | 4.64 |

Note: NMAD (Normalized Median Absolute Deviation) is a robust accuracy metric less sensitive to outliers than RMSE.

# 2. Accuracy by Slope Category

Analysis of height accuracy stratified by terrain slope. Steeper slopes typically show larger residuals due to geometric distortions and shadow effects.

| slope_category | count | mean | std | nmad |
| --- | --- | --- | --- | --- |
| Flat (0-5°) | 19,181 | 0.33 | 2.82 | 2.11 |
| Gentle (5-15°) | 15,544 | -0.09 | 5.88 | 5.62 |
| Moderate (15-30°) | 25,231 | 0.71 | 7.72 | 6.91 |
| Steep (>30°) | 4,550 | 2.86 | 11.16 | 9.03 |

# 3. Accuracy by Land Cover Type

Analysis of height accuracy stratified by land cover classification. Different surface types exhibit varying radar backscatter properties.

| land_cover | count | mean | std | nmad |
| --- | --- | --- | --- | --- |
| Beaches, dunes, sands | 1,614 | -1.01 | 1.98 | 1.70 |
| Broad-leaved forest | 13,432 | 2.78 | 9.92 | 8.02 |
| Complex cultivation patterns | 2,478 | 0.80 | 3.15 | 2.28 |
| Coniferous forest | 1,260 | 1.44 | 6.41 | 5.93 |
| Discontinuous urban fabric | 5,082 | 1.15 | 3.33 | 2.33 |
| Land principally occupied by agriculture, with significant areas of natural vegetation | 11,092 | -0.49 | 7.19 | 6.84 |
| Mixed forest | 254 | 0.19 | 5.79 | 5.07 |
| Olive groves | 1,839 | -4.55 | 4.54 | 2.88 |
| Pastures | 5,927 | -1.68 | 5.42 | 4.35 |
| Vineyards | 21,563 | 0.65 | 4.18 | 3.49 |

# 5. Radar Geometry and Shadow Statistics

Statistics for different radar viewing geometry categories, including shadow and layover zones.

| Category | Count | Bias (m) | RMSE (m) | NMAD (m) |
| --- | --- | --- | --- | --- |
| optimal | 45,485 | +0.65 | 6.22 | 4.33 |
| acceptable | 58,689 | +0.62 | 6.58 | 4.81 |
| steep | 10 | +8.54 | 11.12 | 6.40 |
| layover | 5,807 | -0.12 | 6.80 | 5.67 |

# 7. Key Figures

## Residual Distributions

## Accuracy by Land Cover

## Accuracy by Elevation

## Terrain Slope Analysis

## Coverage and Voids

## Radar Geometry Analysis

## Summary Dashboard